1. What is AWS Elastic Load Balancing (ELB)?

Answer: AWS ELB automatically distributes incoming traffic across multiple targets (EC2, containers, IPs, Lambda) to ensure high availability and fault tolerance.

2. What are the types of Elastic Load Balancers?

Answer:

- Classic Load Balancer (CLB): Layer 4 & 7 (legacy)

- Application Load Balancer (ALB): Layer 7, HTTP/HTTPS, advanced routing

- Network Load Balancer (NLB): Layer 4, high performance, static IP

- Gateway Load Balancer (GWLB): For third-party virtual appliances (firewalls, etc.)

3. What is the difference between ALB and NLB?

Answer:

- ALB: Works at Layer 7, supports host/path-based routing, WebSockets.

- NLB: Works at Layer 4, supports TCP/UDP, static IP, ultra-low latency.

4. What is a Target Group in ELB?

Answer: A logical grouping of targets (EC2, IPs, Lambda) that receive traffic from the load balancer.

5. How does ELB perform health checks?

Answer: ELB periodically sends health check requests (HTTP/TCP) to targets.

- Unhealthy targets are removed from routing until they pass.

6. What is the difference between Cross-Zone Load Balancing and AZ Balancing?

Answer:

- Cross-Zone LB: Distributes traffic evenly across all targets in all AZs.

- AZ Balancing: Routes traffic only to targets in the AZ of the load balancer node.

7. Can ELB handle SSL termination?

Answer: Yes. ALB, NLB (TLS), and CLB support SSL/TLS termination to offload encryption from backend servers.

8. Does ELB have a static IP address?

Answer:

- NLB: Yes, supports static IP.

- ALB/CLB: No, but you can use Route 53 with DNS.

9. What is Sticky Session in ELB?

Answer: A feature (session affinity) where requests from a client are routed to the same backend instance using cookies.

10. Can ELB integrate with Auto Scaling?

Answer: Yes, ELB works seamlessly with Auto Scaling Groups to add/remove instances dynamically.

11. What is the difference between Listener and Rule in ALB?

Answer:

- Listener: Listens for incoming traffic on a port/protocol.

- Rule: Defines how to route traffic based on conditions (host, path, headers).

12. What is Host-Based vs Path-Based Routing?

Answer:

- Host-Based: Routes traffic based on domain name.

- Path-Based: Routes traffic based on URL path.

13. What is an Idle Timeout in ELB?

Answer: The time an idle connection is kept open.

- Default: 60 seconds for ALB/CLB.

14. Can ELB handle WebSockets?

Answer: Yes, ALB supports WebSockets.

15. How does ALB support microservices?

Answer: By using host/path-based routing, ALB can direct traffic to different target groups for microservices.

16. Can ELB perform redirect or fixed responses?

Answer: Yes, ALB supports HTTP redirects and fixed responses without sending traffic to the backend.

17. What is Deregistration Delay in ELB?

Answer: Time ELB waits before removing a target during scale-in to allow in-flight requests to complete.

18. How do you secure an ELB?

Answer:

- Use Security Groups, WAF, TLS, and Access Logs.

- Restrict access to backend only from ELB.

19. Does ELB support IPv6?

Answer: Yes, ALB and NLB support dual-stack (IPv4/IPv6).

20. Can ELB send logs?

Answer: Yes, Access Logs can be stored in S3 for analysis.

21. Scenario: You need to route traffic to different applications based on the URL path.

Answer: Use ALB with Path-Based Routing.

22. Scenario: You need a load balancer for TCP traffic with high performance.

Answer: Use NLB.

23. Scenario: Clients need to connect to your backend using a static IP.

Answer: Use NLB or assign an Elastic IP (for GWLB).

24. Scenario: You have long-lived connections (e.g., streaming).

Answer: Use NLB (supports millions of connections) or increase Idle Timeout in ALB.

25. Scenario: Ensure traffic is always encrypted end-to-end.

Answer: Enable TLS on ELB and enforce TLS between ELB → backend.

26. Scenario: Route some traffic to a new version of the app (canary release).

Answer: Use Weighted Target Groups with ALB.

27. Scenario: Scale-down events cause users to lose sessions.

Answer:

- Enable Sticky Sessions.

- Configure Deregistration Delay.

28. Scenario: You need to integrate a third-party firewall appliance with AWS.

Answer: Use Gateway Load Balancer (GWLB).

29. Scenario: You want to filter requests at the load balancer level for security.

Answer:

- Use AWS WAF with ALB.

- Use Listener Rules for basic filtering.

30. Scenario: You must allow only specific IP ranges to access the load balancer.

Answer:

- Use Security Groups and/or Network ACLs.

- ALB Listener rules for IP restriction.

31. Scenario: You need to route traffic to Lambda functions directly.

Answer: Use ALB (supports Lambda targets).

32. Scenario: Need to integrate load balancing with hybrid environments (on-prem + AWS).

Answer: Use NLB with IP targets to route to on-prem resources via VPN/Direct Connect.

33. Scenario: You need to capture detailed request/response logs for compliance.

Answer: Enable ALB Access Logs to S3.

34. Scenario: How to troubleshoot high 5XX errors behind an ALB?

Answer:

- Check target health.

- Verify app logs.

- Enable ELB access logs for investigation.

35. Scenario: Some requests are being sent to unhealthy instances.

Answer:

- Verify Health Check Path.

- Ensure correct Health Check Grace Period.

36. Scenario: You want to reduce costs by serving static content without backend.

Answer:

- Use CloudFront or S3 Static Website.

- Alternatively, use ALB Fixed Responses.

37. Scenario: You need a load balancer that supports UDP traffic.

Answer: Use NLB, as ALB/CLB do not support UDP.

38. Scenario: You need to allow WebSocket connections for a real-time app.

Answer: Use ALB, which supports WebSocket.

39. Scenario: You want to log client IPs behind the load balancer.

Answer:

- ALB includes X-Forwarded-For header.

- Ensure backend reads this header.

40. Scenario: You want to distribute traffic evenly across multiple AZs even if some AZs have fewer targets.

Answer: Enable Cross-Zone Load Balancing.